Instructions for Use of Disposable Viral Sampling Swab Kit

[Product Name]  Disposable Viral Sampling Swab Kit

[Scope of Application]  Used for the collection, transportation and preservation of samples.

[Function and Purpose]  The disposable virus sampling swab kit supports the whole-process operations of environmental samples from collection, preservation to transportation to the laboratory. It reduces the contamination risks in multiple links such as specimen collection and storage, and ensures effective protection of specimens before analysis.

[Main Components]  The disposable viral sampling swab kit consists of sampling swabs and matched nucleic acid protectant.

[Instructions for Use]  Take out the swab to collect required samples. Carefully unscrew the sampling tube pre-filled with nucleic acid protectant, then quickly place the sample-contaminated swab into the tube. Align the breaking point with the inner wall of the tube and press to break off the shaft, leaving the swab head immersed in the liquid inside the sampling tube. Finally, tighten the tube cap and shake gently left and right for 3 times.

[Storage and Shelf Life]  The shelf life of the product is 2 years. After sample collection, place the specimen in the sampling tube with nucleic acid protectant for the following storage stability:

• Stable storage for 3 days at 37°C

• Stable storage for 7 days at 18–25°C

• Storage for up to 4 weeks at 2–8°C

• Long-term storage at -20°C or -80°C

Product Specifications

| Product Name | Packaging Type | Item No. | Kit Composition | Minimum Packaging | Sales Packaging | Units/CTN |
| --- | --- | --- | --- | --- | --- | --- |
| Disposable Viral Sampling Swab Kit | Single-person Kit | SADS01 | 1 short flocked swab 1 sampling tube (with nucleic acid protectant) | 50 pcs/bag | 20 bags/ctn | 1000 |
|  | 5-in-1 Pooled Kit | SADS02 | 5 short flocked swabs 1 sampling tube (with nucleic acid protectant) | 25 pcs/bag | 20 bags/ctn | 500 |
|  | Special Sampling Kit for Pig Farms | SADS03 | 50 long flocked swabs 50 sampling tubes (with nucleic acid protectant) | 50 pcs/bag | 20 bags/ctn | 1000 |